MOBILE EQUIPMENT OPERATOR SAFETY

- Inspect equipment for safety defects by using and completing the prescribed pre-operating checks, before placing the equipment in service. Remove from service any equipment with a defect that could affect safety in any manner. Tag the defective equipment in accordance with company policy and report the defect to the supervisor.

- Every operator of a piece of mobile equipment is required to have a certificate of training for that equipment. The certificate verifies that the operator is trained in all safe operating procedures for that piece of equipment.

- Operators of small vehicles (pickups and vans) must complete the driver's permit training and obtain a site driver's license.

- Fasten seat belts whenever the equipment is in motion.

- Always operate equipment at a prudent speed. Do not endanger personnel through careless handling of the machine. Slow down for curves and during abnormal weather or operating conditions.

- Before leaving any equipment unattended, make sure that attachments are fully lowered to the ground and that parking brakes are set. Always use wheel chocks unless parked at a ready line or parking space with a wheel depression.

- Be certain that all persons and equipment are at a safe distance before starting or moving mobile equipment. Sound the horn before starting or moving equipment. The horn signals are applied:

- Once prior to stopping.

- Twice prior to starting forward.

- Three times prior to starting in reverse.

- Know all emergency shutdown and start-up procedures associated with the equipment.

- Keep the equipment and cab clean and free of extraneous material.

- When entering an intersection, always sound the horn to notify pedestrians and other operators in the vicinity. Entering a building requires a spotter.

- If equipped, ensure that the backup horn is working.