| Work Activity | Related Activity | Comprehensive Hazard Report: Garbage Services Hazards | Risk Ranking Risk = Severity+Probability+Frequency |  |  |  | Controls |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | S | P | F | RR |  |
| Waste Equipment Operation | Completing a Circle Check | Slips Trips and Falls | 1 | 1 | 2 | 4 | - Good House Keeping - snow removal and de-icing agents |
|  |  | Being Struck by Other Equipment | 4 | 1 | 2 | 7 | - Lighting - PPE |
|  |  | Exposure to Exhaust Fumes | 2 | 1 | 1 | 4 | - Idle only in well ventilated areas - use ventilation system if vehicle is in the shop |
|  | Operating | Inexperienced Operator | 3 | 2 | 1 | 6 | - Licensed Drivers - In house training |
|  |  | Traffic on Public Roadways | 1 | 3 | 3 | 7 | - Use of Spotters to stop traffic if required - Amber Strobe lights /emergency flashers and work lights - Audible Back Up beepers |
|  |  | Bystanders / Construction Crews | 4 | 2 | 2 | 8 | - PPE - Spotters when required - Amber Strobe lights /emergency flashers and work lights - Audible Back Up beepers |
|  |  | Sprains/Strains climbing in and out | 3 | 3 | 3 | 9 | - SWP Garbage Collection |
|  |  | Road and Ground Conditions | 3 | 2 | 2 | 7 | - Trained Drivers - SWP Delivering/Removing Containers - Use de-icing agents’ - Adjust routing to accommodate slippery or impassible roadways - Use Planks to protect Driveways |
|  |  | Weather | 1 | 3 | 2 | 6 | - PPE - Extra Clothes i.e. Gloves - Sunblock - Water |
|  |  | Mechanical Failure | 1 | 1 | 1 | 3 | - PPE - Flare Kits - Fire Extinguisher - Communication / Emergency Contact Numbers |
|  |  | Oversized/Overweight Loads | 1 | 1 | 1 | 3 | - SWP picking up containers - Load must be secured before we travel - Remove Material if required |
|  | Backing Up | Striking Object or Individual | 4 | 3 | 3 | 10 | - Use Spotter when visibility is compromised - Audible Back up Beeper - Back Up Cameras |
|  |  | Overhead Power Lines | 4 | 1 | 1 | 6 | - SWP Power Line Hazards - Permit from Utility if Closer than 18 Ft - Power Line Hazard Training - Emergency Response Plan for Accidental Contact - First Aid - Fire Extinguisher - Use of a Spotter when required |
|  | Container Deliveries Removals | Drop Offs / Deliveries | 4 | 2 | 2 | 8 | - Trained Operators - SWP Container Delivery - Spotters to stop traffic - PPE |
|  |  | Pick Ups / Removals | 4 | 2 | 2 | 8 | - Trained Operators - SWP Container Pick Up - Spotters to stop traffic - PPE |
|  |  | Poor Ground Conditions | 4 | 2 | 2 | 8 | - Informal Hazard Assessment - Only Park and operate on firm solid level ground - Beware of soft narrow shoulders - Beware of excavated area’s |
|  | Unloading | Overhead Power Line Contact | 4 | 1 | 1 | 6 | - SWP Unloading Waste Equipment Roll Off, Compactor Trucks |
|  |  | Blowing and Falling Debris | 2 | 2 | 2 | 6 | - PPE - Access Load before opening door - Open One Door at a time and stand away from the pinch points |
|  |  | Cuts, Boot Punctures | 2 | 2 | 2 | 6 | - PPE - Aware of Surroundings - Vaccinations |
|  |  | Being Struck by other Equipment | 4 | 1 | 2 | 7 | - PPE - Follow Site Rules |
|  |  | Roll Over's | 4 | 1 | 1 | 6 | - Check to ensure load is balanced - Ground must be solid and level - Remain inside cab while dumping |
|  | Tarping Loads | Falling Materials | 2 | 2 | 2 | 6 | - PPE - Inspect load at ground level before picking up - Remove all hanging debris |
|  |  | Overhead Obstructions | 3 | 2 | 2 | 7 | - PPE - Hazard Assessment ie roof eves, wires |
|  | Loading: Residential Collection | Slip Trips and Falls | 3 | 3 | 3 | 9 | - PPE - Situational Awareness (Broken Curb, ice, catch basins, sand covered asphalt etc) - SWP Residential Garbage Collection |
|  |  | Needle Pricks / Cuts and Lacerations | 4 | 2 | 3 | 9 | - PPE - Vaccinations - SWP Needle Pricks, Residential Garbage Collection |
|  |  | Flying Debris | 2 | 1 | 1 | 4 | - PPE - Training |
|  | Compacting Garbage in Truck | Pinch Points | 4 | 2 | 1 | 7 | - Machine Guarding - Training - Labels - Lock out Procedures |
|  |  | MSI, Repetitive Strain | 3 | 3 | 3 | 9 | - Job Procedure Residential - Garbage Collection |
|  | Fueling | Spills | 1 | 1 | 1 | 3 | - Spill Kits - PPE |
|  |  | Fire/Explosions | 1 | 1 | 1 | 3 | - Fire Extinguishers - Emergency Contact Numbers |
|  |  | Vapors/Gas exposure | 1 | 1 | 1 | 3 | - Well ventilated areas |

Comprehensive Hazard Assessment

Risk Ranking

Severity

+0 No Injury

+1 Minor Injury requiring first aid

+2 Medical treatment with no lost time beyond the day of injury and minor property damage ($500 or less)

+3 Lost Time Injury or significant property damage (greater than $500 but less than $5000)

+4 Permanent Disability/Fatality(ies) or major property damage (greater than $5000)

Probability

+1 Exposure to the uncontrolled hazard is unlikely to occur

+2 Exposure to the uncontrolled hazard could occur

+3 Exposure to the uncontrolled hazard will occur

Frequency

| Number of Persons Exposed | Number of times persons may be exposed to or have contact with the hazard. |  |  |
| --- | --- | --- | --- |
|  | Less than daily | Few times a day | Many time a day |
| Few | +1 | +1 | +2 |
| Moderate | +1 | +2 | +3 |
| Many | +2 | +2 | +3 |

Risk = Severity + Probability + Frequency

Low Risk 0-2           Medium Risk 3-5           High Risk 6-10