SCAFFOLDS

All scaffolds must be installed, inspected, maintained, altered, and dismantled in accordance with Provincial OHS Regulations and applicable standards and/or manufacturer’s specifications under the direct supervision of qualified workers.

A scaffold or scaffolding is a temporary work platform and supporting structure used for supporting workers and/or materials. When selecting the type of scaffolding to be used, consideration must be given to height requirements, type and duration of work, weather conditions, and weight of workers, materials, and equipment. A scaffold must be able to support at least four times the weight of the maximum load intended and must be inspected each day prior to use by a qualified person.

Do:

- Ensure that a scaffold is in safe condition prior to use (check tag).

- Ensure planks, decks, toe boards and guardrails are installed and secure.

- Use a hoist or rope to move materials to upper levels.

- Ensure that there is a safe entrance to work levels.

- Ensure cross bracing is installed at all levels and locking device and ties are secured.

- Ensure the base is sound, level, and adjusted.

- Use proper fall protection if required.

Do not:

- Carry materials when climbing scaffolding.

- Force braces to fit.

- Climb or stand on cross braces or guardrails.

- Work on scaffolding during storms or high winds.

- Jump from planks or platforms.

- Rest materials or equipment on guardrails.

- Use ladders or makeshift devices on top of scaffolds to increase height.

- Work below a scaffold without head protection.

- Use scaffolds near power lines or other conductive materials.